Representative Hardware Performance Summary

Instead of listing every repeated value in one wide table, the results are separated into three compact views: model quality, hardware feasibility, and timing. All four students share the same deployment profile: 96 x 96 x 3 INT8 input, 0.00553M parameters, 7.81M FLOPs, 22,728-byte model size, and 204,800-byte tensor arena.

A. Model Quality View

| Model | Accuracy (%) | F1-score (%) | Observed output | Interpretation |
| --- | --- | --- | --- | --- |
| ConvNeXt student | 44.90 | 43.48 | c8 | Weakest accuracy |
| EfficientNet-B0 student | 52.28 | 50.30 | c6 | Moderate |
| MobileNetV3-Small student | 54.92 | 54.38 | c2 / c1 | Balanced |
| ResNet50 student | 59.28 | 58.71 | c8 | Best validation score |

Table 1 Compact performance view of the four distilled students.

B. Hardware Feasibility View

| Hardware | Memory | Status | Performance | FPS |
| --- | --- | --- | --- | --- |
| ESP32 DevKit v1 | 520 KB SRAM | Runs | Slow | 0.41 |
| ESP32-S3 / M5Stack CoreS3 | 320 KB SRAM + 8 MB PSRAM | Runs | Slow in tested profile | 0.41 |
| Raspberry Pi Pico / RP2040 | 264 KB SRAM | Runs | Fastest measured profile | 23.04 |

Table 2 Compact hardware feasibility view.

C. Timing Matrix

| Model group | ESP32 latency | ESP32 FPS | ESP32-S3/M5Stack latency | ESP32-S3/M5Stack FPS | RP2040 latency | RP2040 FPS |
| --- | --- | --- | --- | --- | --- | --- |
| All four 96 RGB students | 2419.6 ms | 0.41 FPS | 2419.6 ms | 0.41 FPS | 43.4 ms | 23.04 FPS |

Table 3 Timing matrix for the shared 96 RGB student deployment profile.

Key reading: ResNet50-derived distillation gives the best validation result, while RP2040 gives the fastest measured timing profile in the current hardware tests. ESP32 and ESP32-S3/M5Stack are feasible but slower in the tested 96 RGB student configuration.