Four Distilled Student Models: Performance on Each Hardware

The table summarizes the four distilled student models on the tested hardware targets. All four models share the same TinyML student architecture, input size, model size, tensor arena, parameter count, and FLOPs; their classification behaviour differs because each student was distilled from a different teacher.

| Model | Hardware | Hardware memory | Input | Parameters | FLOPs | Size | Tensor arena | Latency (ms) | FPS | Accuracy (%) | Observed output | Confidence | Performance note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ConvNeXt student | ESP32 DevKit v1 | 520 KB SRAM, no PSRAM | 96 x 96 x 3 INT8 | 0.00553 M | 7.81 M | 22,728 bytes / 0.0217 MB | 204,800 bytes | 2419.6 | 0.41 | 44.90 | c8_hair_and_makeup | 0.6700 | Runs, but slow |
| ConvNeXt student | ESP32-S3 DevKitC / M5Stack CoreS3 profile | 320 KB internal SRAM + 8 MB PSRAM | 96 x 96 x 3 INT8 | 0.00553 M | 7.81 M | 22,728 bytes / 0.0217 MB | 204,800 bytes | 2419.6 | 0.41 | 44.90 | c8_hair_and_makeup | 0.6700 | Runs with PSRAM available; same model behaviour as ESP32 profile in tested firmware |
| ConvNeXt student | Raspberry Pi Pico / RP2040 | 264 KB SRAM | 96 x 96 x 3 INT8 | 0.00553 M | 7.81 M | 22,728 bytes / 0.0217 MB | 204,800 bytes | 43.4 | 23.04 | 44.90 | c8_hair_and_makeup | 0.6700 | Runs within SRAM in tested setup |
| EfficientNet-B0 student | ESP32 DevKit v1 | 520 KB SRAM, no PSRAM | 96 x 96 x 3 INT8 | 0.00553 M | 7.81 M | 22,728 bytes / 0.0217 MB | 204,800 bytes | 2419.6 | 0.41 | 52.28 | c6_drinking | 0.3974 | Runs, but slow |
| EfficientNet-B0 student | ESP32-S3 DevKitC / M5Stack CoreS3 profile | 320 KB internal SRAM + 8 MB PSRAM | 96 x 96 x 3 INT8 | 0.00553 M | 7.81 M | 22,728 bytes / 0.0217 MB | 204,800 bytes | 2419.6 | 0.41 | 52.28 | c6_drinking | 0.3974 | Runs with PSRAM available; same model behaviour as ESP32 profile in tested firmware |
| EfficientNet-B0 student | Raspberry Pi Pico / RP2040 | 264 KB SRAM | 96 x 96 x 3 INT8 | 0.00553 M | 7.81 M | 22,728 bytes / 0.0217 MB | 204,800 bytes | 43.4 | 23.04 | 52.28 | c6_drinking | 0.3974 | Runs within SRAM in tested setup |
| MobileNetV3-Small student | ESP32 DevKit v1 | 520 KB SRAM, no PSRAM | 96 x 96 x 3 INT8 | 0.00553 M | 7.81 M | 22,728 bytes / 0.0217 MB | 204,800 bytes | 2419.6 | 0.41 | 54.92 | c2_talking_phone_right / c1_texting_right | 0.4596 / 0.9996 | Runs, but slow |
| MobileNetV3-Small student | ESP32-S3 DevKitC / M5Stack CoreS3 profile | 320 KB internal SRAM + 8 MB PSRAM | 96 x 96 x 3 INT8 | 0.00553 M | 7.81 M | 22,728 bytes / 0.0217 MB | 204,800 bytes | 2419.6 | 0.41 | 54.92 | c2_talking_phone_right / c1_texting_right | 0.4596 / 0.9996 | Runs with PSRAM available; same model behaviour as ESP32 profile in tested firmware |
| MobileNetV3-Small student | Raspberry Pi Pico / RP2040 | 264 KB SRAM | 96 x 96 x 3 INT8 | 0.00553 M | 7.81 M | 22,728 bytes / 0.0217 MB | 204,800 bytes | 43.4 | 23.04 | 54.92 | c2_talking_phone_right / c1_texting_right | 0.4596 / 0.9996 | Runs within SRAM in tested setup |
| ResNet50 student | ESP32 DevKit v1 | 520 KB SRAM, no PSRAM | 96 x 96 x 3 INT8 | 0.00553 M | 7.81 M | 22,728 bytes / 0.0217 MB | 204,800 bytes | 2419.6 | 0.41 | 59.28 | c8_hair_and_makeup | 0.7442 | Runs, but slow |
| ResNet50 student | ESP32-S3 DevKitC / M5Stack CoreS3 profile | 320 KB internal SRAM + 8 MB PSRAM | 96 x 96 x 3 INT8 | 0.00553 M | 7.81 M | 22,728 bytes / 0.0217 MB | 204,800 bytes | 2419.6 | 0.41 | 59.28 | c8_hair_and_makeup | 0.7442 | Runs with PSRAM available; same model behaviour as ESP32 profile in tested firmware |
| ResNet50 student | Raspberry Pi Pico / RP2040 | 264 KB SRAM | 96 x 96 x 3 INT8 | 0.00553 M | 7.81 M | 22,728 bytes / 0.0217 MB | 204,800 bytes | 43.4 | 23.04 | 59.28 | c8_hair_and_makeup | 0.7442 | Runs within SRAM in tested setup |

Table 1 Per-hardware performance summary for the four distilled student models.

Notes: ESP32-S3 DevKitC and M5Stack CoreS3 are grouped because the tested configuration uses the same ESP32-S3 class deployment path. Raspberry Pi Pico/RP2040 latency is listed from the RP2040 run. ESP32/ESP32-S3 latency values use the recorded 96 x 96 RGB student timing from the hardware-oriented logs. The observed output column records the project image tests; full accuracy is from the driver-wise validation summaries.