# ESP32 TinyML Driver Distraction Detection - Daily Progress Report

Date: July 1, 2026

# Objective

The goal was to run a driver distraction image classification model on microcontroller hardware and in Wokwi simulation, control LEDs based on the classification result, and benchmark memory and latency behavior for research documentation.

# Project Journey

| Stage | Problem / Observation | Fix / Action | Result |
| --- | --- | --- | --- |
| Original 224 RGB model | Model file was only 179,868 bytes, but runtime memory was much larger. | Inspected TFLite input/output and tensor arena behavior. | AllocateTensors needed about 4,903,680 bytes, proving RAM was the limiting factor. |
| ESP32-S3 PSRAM setup | PSRAM initially showed ID read error and Free PSRAM was 0. | Enabled ESP32-S3 with 8 MB PSRAM in Wokwi diagram and allocated tensor arena in PSRAM. | Free PSRAM became about 8,386,035 bytes and the large model could run. |
| No Wokwi terminal output | The program printed before the terminal attached or serial mode was wrong. | Added boot delay, Serial wait, and later switched Wokwi S3 from USB CDC to UART serial. | Terminal output became visible and stable. |
| Firmware mismatch | Wokwi used ESP32-S3 board but loaded firmware built for ESP32 DevKit V1. | Added separate PlatformIO environment named wokwi-esp32-s3 and pointed wokwi.toml to that firmware. | Boot ROM reset loop was fixed. |
| Normal ESP32 test | Real board was ESP32 DevKit V1 with no PSRAM. 256 KB tensor arena failed. | Changed PlatformIO board to esp32doit-devkit-v1 and reduced arena to 102,400 bytes. | The optimized model ran using internal SRAM. |

# Model Comparison

| Model | Input | Model Size | Tensor Arena | Prediction | Accuracy Observation | Latency |
| --- | --- | --- | --- | --- | --- | --- |
| Original larger TFLite | 224 x 224 x 3 | 179,868 bytes | 6,291,456 bytes used in final PSRAM run; earlier required about 4,903,680 bytes | c5_operating_radio | Correct for the test image; best accuracy among tested models | Very slow in Wokwi; required ESP32-S3 PSRAM |
| 96 grayscale INT8 | 96 x 96 x 1 | 30,400 bytes | 262,144 bytes first; later 102,400 bytes could run on ESP32 | c3_texting_left | Faster and smaller, but misclassified expected c5 image | About 1.5 s estimated in earlier Wokwi run |
| 64 grayscale INT8 | 64 x 64 x 1 | 20,952 bytes | 102,400 bytes | c0_safe_driving | Fastest, but lost too much detail and misclassified expected c5 image | Wokwi sim-latency 4909 ms; estimated actual 245.4 ms; real ESP32 measured around 301 ms before label change |

# Hardware / Simulation Comparison

| Platform | RAM / PSRAM | Firmware Target | Result | Important Finding |
| --- | --- | --- | --- | --- |
| Wokwi ESP32-S3 | Internal SRAM plus configurable PSRAM in diagram | wokwi-esp32-s3 | Runs 64 x 64 model; large model also ran when arena used PSRAM | Good for demonstration, but latency is slower than hardware and must be treated as simulation latency. |
| Real ESP32 DevKit V1 | No PSRAM; internal SRAM only | esp32doit-devkit-v1 | Runs 64 x 64 INT8 model with 102,400 byte arena | Proves optimized TinyML model can run on a normal ESP32 without PSRAM. |
| M5Stack CoreS3 / ESP32-S3 class board | Expected PSRAM-capable target | m5stack-cores3 or ESP32-S3 target | Useful for larger models | Best hardware choice if accuracy requires 96 RGB or larger tensor arena. |

# Final Working Outputs

| Run | Key Output |
| --- | --- |
| Wokwi 64 x 64 INT8 | Model bytes 20,952; arena 102,400; input 1 x 64 x 64 x 1; sim-latency 4909 ms; estimated actual 245.4 ms; predicted c0_safe_driving; LED red. |
| Real ESP32 DevKit V1 64 x 64 INT8 | Model ran without PSRAM using internal SRAM. Earlier measured actual inference latency was about 301 ms. |
| Original ESP32-S3 PSRAM large model | Model bytes 179,868; arena 6,291,456; input 1 x 224 x 224 x 3; predicted c5_operating_radio; result DISTRACTION; LED green. |

# Next Accuracy Plan

The next step is to improve accuracy without using the borrowed Nemo ConvNeXt model as a teacher, because permission was only for testing. The clean research path is to train a new teacher model in Google Colab using the original 224 x 224 x 3 dataset, then distill that teacher into a small ESP32 student model.

| Experiment | Purpose | Expected Tradeoff |
| --- | --- | --- |
| Train own teacher model | Use MobileNetV2 or EfficientNetB0 on 224 RGB images to get high accuracy. | Higher accuracy, not deployable directly on normal ESP32. |
| Distill 96 grayscale student | Transfer teacher knowledge into a microcontroller-friendly model. | Better accuracy than 64 grayscale while still likely small enough for ESP32-class hardware. |
| Try 96 RGB student | Keep color information for better classification. | Better accuracy, but more RAM and latency. |
| Benchmark all models | Compare model size, arena, flash, RAM, Wokwi latency, real latency, and accuracy. | Gives strong evidence for the research paper. |

# Techniques Used and How They Were Applied

| Technique | Meaning | How it was applied here | Justification |
| --- | --- | --- | --- |
| TinyML | Running machine learning inference on microcontrollers with limited RAM, flash, and CPU. | The driver distraction classifier was converted into TFLite/TFLite Micro style deployment and executed on ESP32/Wokwi with LED output. | This directly supports the research goal of edge AI inference without a PC or cloud connection. |
| TensorFlow Lite Micro | A small inference runtime for microcontrollers. | The firmware used TensorFlowLite_ESP32/TFLite Micro headers to load model.h, allocate a tensor arena, call AllocateTensors, and Invoke the model. | It is suitable for ESP32-class deployment and exposes memory limits clearly. |
| INT8 quantization | Representing weights and activations using 8-bit integers instead of 32-bit floats. | The 96 x 96 and 64 x 64 models used INT8 input/output tensors, reducing model size to 30,400 bytes and 20,952 bytes. | Quantization made the model small enough and fast enough for normal ESP32 internal SRAM. |
| Tensor arena | A fixed memory buffer used by TFLite Micro for intermediate tensors and operator workspace. | The large model needed several MB, while the optimized model ran with a 102,400 byte arena. | Arena size was the key memory bottleneck, more important than the model file size itself. |
| PSRAM allocation | Using external pseudo-static RAM available on some ESP32-S3 boards. | The large 224 RGB model required PSRAM; allocation was moved to PSRAM when internal SRAM was insufficient. | This showed why PSRAM boards can run larger models and normal ESP32 cannot. |
| Internal SRAM optimization | Reducing runtime memory enough to fit inside normal ESP32 RAM. | The final 64 x 64 INT8 model used a 102,400 byte arena and ran on ESP32 DevKit V1 without PSRAM. | This proved deployment on low-cost hardware is possible after optimization. |
| Image preprocessing | Converting input images into the exact shape and type expected by the model. | The firmware converted the stored 224 x 224 x 3 test image into 96 x 96 x 1 or 64 x 64 x 1 grayscale input and quantized it using tensor scale/zero point. | Without matching preprocessing, ESP32 predictions would not match PC/TFLite behavior. |
| Wokwi simulation | Browser/VS Code based microcontroller simulation. | Wokwi was used to test LEDs, serial output, memory behavior, and approximate latency before or alongside hardware testing. | It is useful for demonstration and review, but hardware measurements are still needed for final latency claims. |
| PlatformIO multi-environment build | Maintaining different build targets in one project. | Separate environments were used for esp32doit-devkit-v1 and wokwi-esp32-s3. | This fixed firmware/board mismatch and allowed the same codebase to target simulator and real board correctly. |
| Teacher-student distillation | Training a small student model to imitate a larger teacher model. | This is planned next using a self-trained Colab teacher, not the borrowed Nemo ConvNeXt model. | It is the best route to recover accuracy while keeping a deployable ESP32 model. |
| Transfer learning | Starting from a pretrained model and adapting it to a new task. | The planned teacher will use ImageNet-pretrained MobileNetV2 or EfficientNetB0 and fine-tune on driver distraction classes. | This should improve teacher accuracy with less training time and better feature extraction. |
| Benchmarking | Measuring model size, memory use, latency, and prediction quality. | Results were recorded for large 224 RGB, 96 grayscale INT8, and 64 grayscale INT8 models on Wokwi and ESP32. | These measurements create the evidence needed for comparison in the research paper. |

# Conclusion

Today?s work moved the project from a large model that needed ESP32-S3 PSRAM to an optimized INT8 TinyML model that runs on a normal ESP32 DevKit V1 without PSRAM. The main remaining challenge is accuracy: the 64 x 64 grayscale model is fast but misclassifies the test image. The recommended next step is to train an owned high-accuracy teacher model in Colab, then distill it into a 96 x 96 microcontroller student model and benchmark the new result.